# Projects | nand2tetris

GlenAMacdonald/Nand2TetrisPt1: https://www.coursera.org/learn/build-a-computer/home/welcome (github.com)

# CPU Design -

# Layout:

## Mux16-ARegister – Control Switch:

inputA[16]

- Instruction

inputB[16]

- ALUOutput (M)

C[1] =

- d1 – the d1 bit of the destination instruction indicates that the ARegister should store the results of the computation (Figure 4.4)

  - NOTE: There might need to be some way of indicating that the computation has been completed and needs to be stored – initially assume there no synchronization required.

- Otherwise let the input be the instruction.

Output[16]

- ARegister

## ARegister:

Input[16]

- Mux16-ARegister

C[1] =

- d1 -  the d1 bit of the destination instruction indicates that the ARegister should store the results of the computation (Figure 4.4)

  - NOTE: There might need to be some way of indicating that the computation has been completed and needs to be stored – initially assume there no synchronization required.

- This also needs to load the new instruction, probably at the same time the program counter is incremented.

Output[16]

- Mux16ALU InputA

- AddressM (CPU output)

- ProgramCounter Input

## DRegister:

Input[16]

- ALUOutput (M)

C[1] =

- d2 - the d2 bit of the destination instruction indicates that the DRegister should store the results of the computation (Figure 4.4)

output[16]

- ALUInputX

## writeM:

C[1] =

- d3 - the d3 bit of the destination instruction indicates Memory[A] (memory register addressed by A) should store the results of the computation (Figure 4.4)

## Mux16ALU:

inputA[16]

- ARegisterOutput

inputB[16]

- M

C[1] =

- i  - In the case of a C-instruction, the single a-bit determines whether the ALU will operate on the A register input or on the M input,

  - i == 0 for an A-instruction

  - i == 1 for a C-Instruction

- a ($[12]) (5.3.1)

  - When a == 0 – select A Register

  - When a == 1 – select M as the input

output[16]

- ALU input ‘Y’. – via comparison between figure 2.6 and figure 4.3

## Program Counter. – How does this actually increment??

inputA[16]

- ARegisterOutput

inputB[1]

- reset

C

- Some Load

output

- CPUOut ’PC’

## ALU

inputX[16]

- DRegisterOutput

inputY[16]

- Mux16ALUOutput

inputC’s[6]

- Instruction bits c1-c6

OutputC’s

- Zr[1] – computation resulted in a zero

- Ng[1] – computation resulted in it being less than zero

- Output[16]

## Jump Circuitry:

# Section Overview:

- Section 5.2 gives its exact hardware specification

- Section 5.3 describes how the Hack platform can be implemented from previously built chips, in particular the ALU built in project 2 and the registers and memory systems built in project 3

- Section 5.4 compares the Hack machine with industrial-strength computers, and emphasizes the critical role that optimization plays in the latter.

- Section 5.5 gives an overview of the computer construction project.

## Component Description:

- Data Registers (D): (5.1) - a CPU internal register used for temporary storage

  - If we want to compute (a-b).c it can be used to store the result (a-b).

  - (5.2.1) - The D-register is used solely for storing data values

- Address Registers (A): (5.1) - a CPU internal register used to store an address. Can also be used to as a data register if needed.

  - Set to 17 by the commands: ‘@17, D=A’. Note that @17 sets the next memory location to 17 and then loads that value into the D Register.

  - (5.2.1) the A-register serves three different purposes,

    - storing a data value (just like the D-register),

    - pointing at an address in the instruction memory, or

    - pointing at an address in the data memory.

## Inputs (5.2.4)

### inM[16]

  - From Data Memory (16-bit)

  - Value of Mem[A], the instructions M input

### instruction[16]

  - From Instruction Memory (16-bit)

  - Instruction to execute

### reset[1]

  - reset == 1 continues executing, reset == 0 restarts the current program

## Outputs (5.2.4)

### outM[16]

  - To Data Memory

  - Value to write to Mem[addressM], the instructions ‘M’ output.

### address[15]

  - To Data Memory

  - In which address to write

### writeM[1]

  - Write To Data Memory?

### PC[15]

  - To Instruction Memory

  - Address of next instruction

# Instructions “ixxaccccccdddjjj”

(5.2.1)

## i. i == 0 for an A-instruction, i == 1 for a C-Instruction

### A-Instruction:

The instruction is treated as a 16-bit binary value which is loaded into the A register

### C- Instruction

The instruction is treated as a sequence of control bits that determine which function the ALU should computer, and in which registers the computed value should be stored.

## xx. - These aren’t used

## a[1]

  - (5.3.1) ALU operates on the A register input or M input

  - a == 0, select A register (assumed

  - a == 1, select M input

### c[6]

  - (5.3.1) Control bits – the commands specified for the ALU

  - (4.2.3) – Figure 4.3

### d[3]

  - (5.3.1) Destination – Which Memory Register should accept the ALU’s output M

  - (4.2.3) – Figure 4.4

### j[3]

  - (5.3.1) Jump – Which Instruction Register should be loaded on a Jump command.

  - (4.2.3)

## Jump Operation (5.3.1):

But what if the instruction dictates to effect a “jump n” operation, where n is the address of an instruction located anywhere in the program? According to the Hack language specification, a “jump n” operation is realized using a sequence of two instructions. First, we issue the A-instruction @n, which sets the A register to n; next, we issue a C-instruction that includes a jump directive. According to the language specification, execution always branches to the instruction that the A register points at. Thus, when implementing the CPU, one of our challenges is to come up with a logic gate architecture that realizes the following behavior: if jump then PC = A else PC++. The value of the Boolean expression jump depends on the instruction’s j-bits and on the ALU output.